Your College of Engineering, Coimbatore

Department of Electrical Engineering

A Practical File on

# Basic Electrical Engineering Practical File

B.Tech · Laboratory record

Submitted in partial fulfilment of the practical examination requirements

| SUBMITTED BY | PROGRAMME | SUBMITTED TO |
| --- | --- | --- |
| Priya Raghavan Reg. No. 25EE0311 | B.Tech II Sem, EE Lab Batch 3 | Prof. K. Subramanian |

Session 2025–26

CERTIFICATE This is to certify that Priya Raghavan, a student of B.Tech II Sem, EE Lab Batch 3, has satisfactorily completed the practical work in Basic Electrical Engineering prescribed by the Department of Electrical Engineering during the academic session 2025–26. The experiments recorded in this file were performed by the student in the laboratory under my supervision, and the observations and results are the student's own. ______________________; ______________________; ______________________. Prof. K. Subramanian Subject Teacher; External Examiner Date: __________; Dr. L. Narayanan Head of Department.

# Acknowledgement

I would like to express my sincere gratitude to my subject teacher, Prof. K. Subramanian, for guidance and encouragement throughout the practical sessions, and for patiently clearing my doubts in the laboratory.

I am also thankful to Dr. L. Narayanan, Head of Department, for providing a well-equipped laboratory, and to the laboratory staff for their help in arranging the apparatus for every experiment.

Finally, I thank my parents and classmates for their constant support while I completed this practical file.

Priya Raghavan

# Index

| S. No. | Name of the Experiment | Date | Remarks | Teacher's Sign |
| --- | --- | --- | --- | --- |
| 1 | Verification of KCL and KVL | 14 Jan 2026 |  |  |
| 2 | Thevenin's theorem | 21 Jan 2026 |  |  |
| 3 | Series RLC resonance | 28 Jan 2026 |  |  |
| 4 | Open-circuit and short-circuit tests on a transformer | 04 Feb 2026 |  |  |

Write the page number of each experiment in the remarks column once the file is complete.

## Laboratory Safety Rules

Check connections before switching on the supply

Never touch live terminals

Use the correct range on meters

Switch off the supply before changing connections

Keep the work bench dry and tidy

# Experiment 1: Verification of KCL and KVL

| Date: 14 Jan 2026 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To verify Kirchhoff's current law and voltage law for a given DC circuit.

### Apparatus / Materials Required

Regulated DC power supply (0–30 V)

Resistors (1 kΩ, 2.2 kΩ, 3.3 kΩ)

Digital multimeters, breadboard and wires

### Theory

Kirchhoff's current law states that the algebraic sum of currents meeting at a node is zero. Kirchhoff's voltage law states that the algebraic sum of all voltages around a closed loop is zero.

ΣI = 0 at a node

ΣV = 0 around a loop

### Procedure

Connect the circuit on the breadboard as per the diagram.

Set the supply to 10 V.

Measure the current in each branch at the node.

Measure the voltage across each element of the loop.

Repeat for 12 V and 15 V and verify both laws.

### Observations

| Supply (V) | I₁ (mA) | I₂ (mA) | I₃ (mA) | I₂ + I₃ (mA) |
| --- | --- | --- | --- | --- |
| 10 | 3.12 | 1.87 | 1.25 | 3.12 |
| 12 | 3.75 | 2.25 | 1.50 | 3.75 |
| 15 | 4.68 | 2.81 | 1.87 | 4.68 |

Observation table (sample readings — replace with your own)

### Result

Result The sum of currents entering the node equals the sum leaving, and the loop voltages add up to zero; KCL and KVL are verified (sample result).

### Precautions

Do not exceed the power rating of the resistors.

Use the correct meter ranges.

# Experiment 2: Thevenin's theorem

| Date: 21 Jan 2026 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To verify Thevenin's theorem for a given linear circuit.

### Apparatus / Materials Required

DC supply, resistors and decade resistance box

Multimeters, breadboard and wires

### Theory

Any linear two-terminal network can be replaced by an equivalent circuit consisting of a voltage source Vth in series with a resistance Rth. The load current is the same in both circuits.

IL = Vth / (Rth + RL)

### Procedure

Measure the load current IL in the original circuit.

Remove the load and measure the open-circuit voltage Vth.

Replace the source with its internal resistance and measure Rth.

Build the Thevenin equivalent and measure the load current again.

### Observations

| Quantity | Theoretical | Measured |
| --- | --- | --- |
| Vth (V) | 6.00 | 5.96 |
| Rth (kΩ) | 1.65 | 1.66 |
| IL original (mA) | 2.26 | 2.24 |
| IL equivalent (mA) | 2.26 | 2.23 |

Observation table (sample readings — replace with your own)

### Result

Result The load current in the equivalent circuit matches the original circuit within experimental error; Thevenin's theorem is verified (sample result).

### Precautions

Switch off the supply while changing connections.

# Experiment 3: Series RLC resonance

| Date: 28 Jan 2026 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To study the frequency response of a series RLC circuit and find its resonant frequency and Q-factor.

### Apparatus / Materials Required

Function generator

Resistor (100 Ω), inductor (10 mH), capacitor (0.1 μF)

AC milliammeter / CRO, breadboard

### Theory

In a series RLC circuit, current is maximum at the resonant frequency where inductive and capacitive reactances are equal. The sharpness of resonance is measured by the quality factor.

f₀ = 1 / (2π √(LC))

Q = f₀ / (f₂ − f₁)

### Procedure

Connect R, L and C in series with the function generator.

Keep the input voltage constant and vary the frequency from 1 kHz to 10 kHz.

Note the current at each frequency.

Plot current against frequency and find f₀, f₁ and f₂.

### Observations

| f (kHz) | I (mA) |
| --- | --- |
| 2 | 6.1 |
| 4 | 18.4 |
| 5 | 42.0 |
| 5.03 | 43.5 |
| 6 | 24.8 |
| 8 | 10.2 |

Observation table (sample readings — replace with your own)

### Result

Result Resonant frequency ≈ 5.0 kHz; Q-factor ≈ 3.2 (sample result).

### Precautions

Keep input amplitude constant at every frequency.

# Experiment 4: Open-circuit and short-circuit tests on a transformer

| Date: 04 Feb 2026 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To perform open-circuit and short-circuit tests on a single-phase transformer and determine its efficiency and regulation.

### Apparatus / Materials Required

1 kVA, 230/115 V single-phase transformer

Autotransformer, wattmeters (LPF and UPF)

Ammeters and voltmeters

### Theory

The open-circuit test gives the core (iron) losses, and the short-circuit test gives the full-load copper losses. These are used to predict efficiency and regulation without loading the transformer.

η = output / (output + Pi + x² Pcu) × 100

### Procedure

For the OC test, apply rated voltage to the LV side with the HV side open; note V, I and W.

For the SC test, short the LV side and apply reduced voltage to the HV side until rated current flows; note V, I and W.

Calculate the equivalent circuit parameters, efficiency and regulation.

### Observations

| Test | V (V) | I (A) | W (W) |
| --- | --- | --- | --- |
| Open circuit | 115 | 0.42 | 18 |
| Short circuit | 12.5 | 4.35 | 42 |

Observation table (sample readings — replace with your own)

### Result

Result Iron loss = 18 W, full-load copper loss = 42 W; full-load efficiency at unity power factor ≈ 94.3% (sample result).

### Precautions

Use an autotransformer starting from zero.

Use an LPF wattmeter for the OC test.

# Bibliography

Laboratory manual issued by the department / school.

Prescribed textbook for the subject (latest edition).

Class notes and practical demonstrations by the subject teacher.