A PRACTICAL FILE ON # Engineering Physics Practical File B.Tech First Year · Laboratory record Submitted in partial fulfilment of the practical examination requirements BACHELOR OF TECHNOLOGY in B.Tech I Sem, Section B Submitted by Aditya Kumar Enrolment No. 25BT0142; Programme B.Tech I Sem, Section B. Department of Applied Physics Your Institute of Technology, Nagpur Session 2025–26

CERTIFICATE This is to certify that Aditya Kumar, a student of B.Tech I Sem, Section B, has satisfactorily completed the practical work in Engineering Physics prescribed by the Department of Applied Sciences during the academic session 2025–26. The experiments recorded in this file were performed by the student in the laboratory under my supervision, and the observations and results are the student's own. ______________________; ______________________; ______________________. Dr. S. N. Bhat Subject Teacher; External Examiner Date: __________; Dr. P. Raman Head of Department.

# Acknowledgement

I would like to express my sincere gratitude to my subject teacher, Dr. S. N. Bhat, for guidance and encouragement throughout the practical sessions, and for patiently clearing my doubts in the laboratory.

I am also thankful to Dr. P. Raman, Head of Department, for providing a well-equipped laboratory, and to the laboratory staff for their help in arranging the apparatus for every experiment.

Finally, I thank my parents and classmates for their constant support while I completed this practical file.

Aditya Kumar

# Index

| S. No. | Name of the Experiment | Date | Remarks | Teacher's Sign |
| --- | --- | --- | --- | --- |
| 1 | Newton's rings | 08 Jul 2025 |  |  |
| 2 | Wavelength using a diffraction grating | 22 Jul 2025 |  |  |
| 3 | Energy band gap of a semiconductor | 05 Aug 2025 |  |  |

Write the page number of each experiment in the remarks column once the file is complete.

## Laboratory Safety Rules

Check connections before switching on the supply

Never touch live terminals

Use the correct range on meters

Switch off the supply before changing connections

Keep the work bench dry and tidy

# Experiment 1: Newton's rings

| Date: 08 Jul 2025 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To determine the wavelength of sodium light by Newton's rings.

### Apparatus / Materials Required

Plano-convex lens of large radius and optically flat glass plate

Sodium vapour lamp

Travelling microscope and glass plate inclined at 45°

Spherometer

### Theory

When a plano-convex lens rests on a flat glass plate, a thin air film of varying thickness forms between them. Interference between rays reflected from the top and bottom of the film produces concentric dark and bright rings. The diameters of the dark rings are related to the wavelength of light and the radius of curvature of the lens.

λ = (D²ₙ₊ₚ − D²ₙ) / (4 p R)

### Procedure

Clean the lens and plate and place them under the microscope with the 45° glass plate.

Illuminate with sodium light and focus the microscope on the rings.

Set the cross-wire on the 16th dark ring on the left and move inward, noting readings for alternate rings.

Continue across the centre to the 16th ring on the right.

Find the diameters and plot D² against ring number.

Measure R with a spherometer.

### Observations

| Ring no. | Left (cm) | Right (cm) | D (cm) | D² (cm²) |
| --- | --- | --- | --- | --- |
| 16 | 5.412 | 4.892 | 0.520 | 0.2704 |
| 12 | 5.380 | 4.928 | 0.452 | 0.2043 |
| 8 | 5.341 | 4.969 | 0.372 | 0.1384 |
| 4 | 5.290 | 5.023 | 0.267 | 0.0713 |

Observation table (sample readings — replace with your own)

### Calculations

Slope of D² vs n = 0.0166 cm² per ring; R = 70 cm (sample).

λ = slope / 4R = 5.93 × 10⁻⁵ cm = 593 nm.

### Result

Result Wavelength of sodium light = 593 nm (sample result; standard value 589 nm).

### Precautions

The lens and plate must be clean and dust-free.

The microscope should move in one direction only to avoid backlash error.

### Viva Questions

Q1. Why is the centre of the ring pattern dark?

At the point of contact the film thickness is zero and the reflected ray undergoes a phase change of π, so the waves interfere destructively.

# Experiment 2: Wavelength using a diffraction grating

| Date: 22 Jul 2025 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To determine the wavelength of the prominent lines of mercury light using a plane transmission grating.

### Apparatus / Materials Required

Spectrometer

Plane transmission grating

Mercury vapour lamp

Spirit level and reading lens

### Theory

A diffraction grating has a large number of equally spaced slits. For normal incidence, bright maxima occur at angles where the path difference between adjacent slits is a whole number of wavelengths.

(a + b) sin θ = n λ

### Procedure

Adjust the spectrometer for parallel rays using Schuster's method.

Set the grating normal to the incident light.

Observe the first-order spectrum on both sides and note the readings for each colour.

Calculate the angle of diffraction for each line.

### Observations

| Colour | Left reading (°) | Right reading (°) | 2θ (°) | θ (°) |
| --- | --- | --- | --- | --- |
| Violet | 193.2 | 166.8 | 26.4 | 13.2 |
| Green | 195.8 | 164.2 | 31.6 | 15.8 |
| Yellow | 196.6 | 163.4 | 33.2 | 16.6 |

Observation table (sample readings — replace with your own)

### Result

Result Wavelengths of the mercury lines were calculated from the measured angles and the grating constant (sample readings above).

### Precautions

Do not touch the ruled surface of the grating.

Read the vernier carefully.

# Experiment 3: Energy band gap of a semiconductor

| Date: 05 Aug 2025 | Page No.: ____ | Teacher's sign: __________ |
| --- | --- | --- |

### Aim

To determine the energy band gap of a semiconductor (germanium) using the four-probe method.

### Apparatus / Materials Required

Four-probe set-up with oven

Germanium crystal

Constant current source and digital voltmeter

Thermometer

### Theory

The resistivity of an intrinsic semiconductor decreases with temperature. Plotting log ρ against 1/T gives a straight line whose slope is proportional to the band gap.

Eg = 2 k × 2.303 × slope

### Procedure

Place the crystal in the oven and connect the four probes.

Pass a constant current and note the voltage at room temperature.

Heat the oven and note the voltage at every 10 °C rise up to about 150 °C.

Calculate resistivity and plot log ρ against 1/T.

### Observations

| T (K) | V (mV) | ρ (Ω cm) | 10³/T (K⁻¹) | log ρ |
| --- | --- | --- | --- | --- |
| 303 | 52.0 | 10.4 | 3.30 | 1.02 |
| 333 | 29.5 | 5.9 | 3.00 | 0.77 |
| 363 | 15.0 | 3.0 | 2.75 | 0.48 |
| 393 | 7.8 | 1.6 | 2.54 | 0.20 |

Observation table (sample readings — replace with your own)

### Result

Result Energy band gap of germanium = 0.70 eV (sample result).

### Precautions

Current should remain constant.

Do not heat beyond the rated temperature.

# Bibliography

Laboratory manual issued by the department / school.

Prescribed textbook for the subject (latest edition).

Class notes and practical demonstrations by the subject teacher.